Zephyr LZ Compression Offload Sub-System

Micro-Architecture Specification (MAS)

Reference SoC Sub-System

| Document Type | Micro-Architecture Specification (MAS) |
| --- | --- |
| Version | 1.0 (WIP) |
| Status | Work In Progress (WIP) |
| Date | September 4, 2026 |
| Author | Mayuresh Piske |

# Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-09-04 | M. Piske | Initial WIP draft for Hackathon 2026 submission. |

# Table of Contents

Revision History	1

Table of Contents	1

1  Introduction	1

1.1  Purpose	1

1.2  Scope	1

1.3  References	1

1.4  Abbreviations	1

2  Micro-Architecture Overview	1

3  Block Micro-Architecture	1

3.1  AXI Interconnect and Arbiter	1

3.2  L2 Cache	1

3.3  Memory Controller	1

3.4  LZ Compression Accelerator (Master M2)	1

3.4.1  Pipeline Stages	1

3.4.2  Match / Effort Level	1

3.5  Input / Output DMA (Masters M0 / M1)	1

3.6  CSR / Register File	1

3.7  Control Module	1

4  Register Map	1

4.1  CTRL Bit-fields (0x000)	1

4.2  CFG Bit-fields (0x004)	1

5  Control Module FSM	1

6  Datapath Flows	1

6.1  Job Fetch	1

6.2  Input Load	1

6.3  Compress	1

6.4  Output Write	1

6.5  Cache Miss / Write-back	1

7  Arbitration Detail	1

8  Performance Numbers	1

9  Error Handling	1

9.1  Detection and Response	1

9.2  ERROR Register Bit-fields (0x040)	1

9.3  Recovery	1

10  Debug and Observability	1

10.1  DBG_CTRL Bit-fields (0x060)	1

10.2  Trace and Observation	1

11  Clock and Reset Domains	1

12  Parameters and Configurability	1

13  Verification and Debug Hooks	1

# 1  Introduction

## 1.1  Purpose

This Micro-Architecture Specification (MAS) details the internal implementation of the Zephyr LZ Compression Offload Sub-System (COSS): block micro-architecture, the compression pipeline, datapaths, the complete firmware register map, the control finite-state machine, arbitration, cache organization, and the performance, error-handling, and debug facilities. It is the implementation companion to the COSS HAS.

## 1.2  Scope

The MAS is the authoritative source for RTL and verification. Where the HAS states what the sub-system does, the MAS specifies how each block is built and how firmware interacts with it at the register and signal level.

## 1.3  References

- Zephyr COSS High-level Architecture Specification (HAS), v0.1.

- ARM AMBA AXI and ACE Protocol Specification (AXI4, AXI4-Lite).

- RFC 1951 – DEFLATE Compressed Data Format (informative).

## 1.4  Abbreviations

| Term | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface (AMBA AXI4) |
| AXI4-Lite | Lightweight, single-beat AXI variant used for register access |
| COSS | Compression Offload Sub-System (this IP) |
| LZ / LZ77 | Lempel-Ziv sliding-window compression algorithm family |
| DEFLATE | LZ77 + Huffman entropy coding (RFC 1951) |
| CSR | Control/Status Register (firmware-visible register space) |
| DMA | Direct Memory Access engine |
| FSM | Finite State Machine |
| QoS | Quality of Service (arbitration priority) |
| RR | Round-Robin (arbitration policy) |
| LRU | Least Recently Used (cache replacement policy) |
| WB | Write-Back (cache write policy) |
| W1C | Write-1-to-Clear register semantics |
| CRC | Cyclic Redundancy Check (integrity) |
| ECC | Error-Correcting Code (SRAM protection) |
| IRQ | Interrupt Request |
| HAS | High-level Architecture Specification |
| MAS | Micro-Architecture Specification |

# 2  Micro-Architecture Overview

Three masters (Input DMA, Output DMA, LZ accelerator) drive an AXI interconnect containing the address decoder and read/write arbiters. The interconnect targets a single memory subordinate fronted by a write-back L2 cache and a memory controller. The control plane — an AXI4-Lite CSR register file and a job-sequencer FSM — consumes job descriptors, drives the datapath, and reports results, performance, errors, and debug state to firmware.

Figure 1 – COSS top-level datapath and control plane.

# 3  Block Micro-Architecture

## 3.1  AXI Interconnect and Arbiter

Implements address decoding, master-to-subordinate routing, and independent write- and read-channel arbitration. Masters expose the full AXI4 channel set (AW, W, B, AR, R); outstanding transactions are tracked by AXI ID for out-of-order responses.

- Policies: round-robin (default) and fixed-priority (Output DMA highest, to drain compressed data and avoid back-pressure), selectable via CFG.ARB_MODE.

- Anti-starvation: a starvation counter promotes a long-waiting master. Per-master grant counts are exported to ARB_GRANTS.

## 3.2  L2 Cache

A 2-way set-associative, write-back, write-allocate cache holds the LZ history window and job descriptors — the most frequently re-read structures. Streaming input/output data may be marked non-cacheable to avoid pollution.

| Parameter | Value (reference) |
| --- | --- |
| Total size | 64 KB |
| Associativity | 2-way set-associative |
| Line size | 64 bytes |
| Sets | 512 |
| Write policy | Write-back, write-allocate |
| Replacement | LRU (1 bit/set) |
| Metadata | Valid + Dirty per line (optional ECC) |
| Primary tenants | History window (32 KB) + job descriptors |

Address decomposition (32-bit): [31:15] tag | [14:6] index (512 sets) | [5:0] offset (64B line).

## 3.3  Memory Controller

Converts cache refill/write-back and streaming DMA bursts into DRAM commands via a bank/row command FSM with a fixed-latency timing model. Separate refill and write-back buffers decouple the datapath from DRAM latency.

## 3.4  LZ Compression Accelerator (Master M2)

The accelerator is the application engine. Its internal pipeline is shown in Figure 2.

Figure 2 – LZ compression accelerator internal pipeline.

### 3.4.1  Pipeline Stages

| Stage | Function |
| --- | --- |
| Input FIFO | Buffers raw bytes streamed in by the Input DMA; decouples DMA bursts from the match rate. |
| Match Finder | Hashes the next few bytes, walks the chained hash-table entries into the history window, and returns the longest match (offset, length). |
| History Window / Dictionary SRAM | Sliding window of recently seen bytes; searched every step and updated as bytes advance (spilled to/filled from cache/DRAM). |
| Hash Table | Head-of-chain pointers per hash bucket for candidate match positions. |
| Token Encoder | Emits a literal token or a (length, distance) copy token based on the match result and effort level. |
| Entropy Coder (optional) | Static/dynamic Huffman coding of the token stream for a DEFLATE-class higher ratio; bypassable in LZ-only mode. |
| CRC / Length Accumulator | Accumulates output CRC and compressed byte count as tokens are emitted. |
| Output FIFO | Buffers the compressed byte stream for the Output DMA. |
| Local Control & Status | Sub-FSM (LOAD → MATCH → EMIT → FLUSH); reports comp_size, CRC, and status to the Control Module. |

### 3.4.2  Match / Effort Level

CFG.LEVEL bounds the chained-search depth: low level = shorter search (higher throughput, lower ratio); high level = deeper search (higher ratio, lower throughput). A minimum-match-length threshold avoids emitting unprofitable short copies.

## 3.5  Input / Output DMA (Masters M0 / M1)

- Input DMA (M0): reads src_len bytes from src_addr in bursts into the input FIFO; sequential, bandwidth-oriented.

- Output DMA (M1): drains the output FIFO to dst_addr in bursts; favored by priority arbitration; enforces dst_cap and flags ERR_DST_OVFL.

## 3.6  CSR / Register File

An AXI4-Lite subordinate implementing the register map in §4. All registers are 32-bit, word-addressed, with defined reset values and access types; reserved bits read zero.

## 3.7  Control Module

The job sequencer decodes control/config, fetches descriptors, orchestrates the DMAs and accelerator, writes back results, aggregates errors and performance events, and generates interrupts. Its FSM is specified in §5.

# 4  Register Map

Offsets are byte offsets from the COSS CSR base. Access: RW, RO, W1C (write-1-to-clear).

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | CTRL | RW | 0x0 | START/doorbell, STOP, SOFT_RESET, CACHE_FLUSH. |
| 0x004 | CFG | RW | 0x0 | MODE, ALGO, LEVEL, WINDOW, ARB_MODE, CACHE_EN, HALT_ON_ERR. |
| 0x008 | STATUS | RO | 0x1 | BUSY, IDLE, ERROR, RING_EMPTY/FULL, active job_id. |
| 0x010 | RING_BASE | RW | 0x0 | Base address of the job descriptor ring in DRAM. |
| 0x014 | RING_HEAD | RO | 0x0 | Consumer index (hardware-updated). |
| 0x018 | RING_TAIL | RW | 0x0 | Producer index (firmware doorbell). |
| 0x01C | RING_SIZE | RW | 0x0 | Number of descriptor entries. |
| 0x020 | PERF_BYTES_IN | RO | 0x0 | Total uncompressed bytes consumed. |
| 0x024 | PERF_BYTES_OUT | RO | 0x0 | Total compressed bytes produced. |
| 0x028 | PERF_JOBS | RO | 0x0 | Completed job count. |
| 0x02C | PERF_CYCLES | RO | 0x0 | Active compression cycles (for throughput). |
| 0x030 | PERF_HITS | RO | 0x0 | Cache hits (history/descriptor). |
| 0x034 | PERF_MISS | RO | 0x0 | Cache misses. |
| 0x038 | ARB_GRANTS | RO | 0x0 | Packed per-master grant counters. |
| 0x040 | ERROR | W1C | 0x0 | Latched error code bits (see §9.2 / §10). |
| 0x044 | ERR_ADDR | RO | 0x0 | Faulting AXI address captured on error. |
| 0x048 | ERR_INFO | RO | 0x0 | Faulting job_id and context. |
| 0x050 | INT_EN | RW | 0x0 | Interrupt enable mask (DONE, ERR, THRESH). |
| 0x054 | INT_STATUS | W1C | 0x0 | Pending interrupts; write 1 to clear. |
| 0x060 | DBG_CTRL | RW | 0x0 | HALT, STEP, BREAK_ON_ERR, FREEZE_CTRS. |
| 0x064 | DBG_STATE | RO | 0x0 | Control FSM state, accelerator sub-state, active job_id. |
| 0x068 | DBG_OBS_SEL | RW | 0x0 | Observation-bus signal-group select. |
| 0x06C | DBG_TRACE | RO | 0x0 | Job trace-buffer read port (pops one entry). |

## 4.1  CTRL Bit-fields (0x000)

| Bits | Field | Description |
| --- | --- | --- |
| [0] | START | Doorbell: begin consuming the job ring. |
| [1] | STOP | Quiesce after the in-flight job. |
| [2] | SOFT_RESET | Self-clearing datapath/FSM reset (CSR config preserved). |
| [3] | CACHE_FLUSH | Flush + invalidate L2; self-clears on completion. |
| [31:4] | Reserved | Reads 0. |

## 4.2  CFG Bit-fields (0x004)

| Bits | Field | Description |
| --- | --- | --- |
| [1:0] | MODE | 0=LZ-only, 1=LZ+Huffman, 2=store, 3=reserved. |
| [3:2] | ALGO | LZ variant / token format select. |
| [6:4] | LEVEL | Effort level 0–7 (match search depth). |
| [9:7] | WINDOW | History-window size select (e.g. 4/8/16/32 KB). |
| [10] | ARB_MODE | 0=round-robin, 1=fixed priority. |
| [11] | CACHE_EN | 1=cache enabled, 0=bypass. |
| [12] | HALT_ON_ERR | 1=halt on error, 0=skip job and continue. |
| [31:13] | Reserved | Reads 0. |

# 5  Control Module FSM

Figure 3 – Job sequencer states.

| State | Action / Transition |
| --- | --- |
| IDLE | Wait for CTRL.START / doorbell. If ring non-empty → FETCH_JOB. |
| FETCH_JOB | Read next descriptor (via cache); validate fields. Bad → ERR; else → LOAD_DATA. |
| LOAD_DATA | Program Input DMA (src_addr, src_len); stream bytes to the accelerator input FIFO. → COMPRESS. |
| COMPRESS | Accelerator runs match/encode; Output DMA drains compressed bytes to dst_addr. On overflow → store fallback / ERR. → WRITE_OUT. |
| WRITE_OUT | Finish output drain and CRC; write comp_size / crc / status into the descriptor. → DONE. |
| DONE | Advance RING_HEAD, update PERF_* counters, push a trace entry, raise DONE/THRESH IRQ. → FETCH_JOB (loop) or IDLE on STOP. |

An error in any state transitions to an error path that aborts the job, latches the ERROR register, and (per CFG.HALT_ON_ERR) halts or advances to the next job.

# 6  Datapath Flows

## 6.1  Job Fetch

Control Module reads the descriptor at RING_HEAD (cache hit expected after warm-up) and validates pointers, length, and flags.

## 6.2  Input Load

Input DMA issues AXI read bursts from src_addr → interconnect arbiter → cache/memory controller → input FIFO. The accelerator consumes bytes as they arrive.

## 6.3  Compress

The match finder searches the history window (resident in cache) for the longest match; the token encoder emits literals or (length, distance) copies; the optional Huffman stage codes the tokens; the CRC/length unit accumulates output stats. Compressed bytes flow to the output FIFO.

## 6.4  Output Write

Output DMA drains the output FIFO to dst_addr with priority arbitration, checking dst_cap. On completion the descriptor result fields are written back.

## 6.5  Cache Miss / Write-back

A history/descriptor miss allocates an LRU way; a dirty victim is drained to the write-back buffer while the refill completes, minimizing stall.

# 7  Arbitration Detail

- Separate write-path (AW/W) and read-path (AR) arbiters, each with its own policy instance.

- Round-robin: rotating grant pointer, bounded wait.

- Fixed-priority: Output DMA > Accelerator (history) > Input DMA, with starvation promotion after a programmable threshold.

- Grant events increment ARB_GRANTS for the fairness demo.

# 8  Performance Numbers

Reference clock 500 MHz, 32 KB window. Throughput ≈ f × bytes-advanced-per-cycle; ratio and rate are derived from PERF counters at runtime.

| Metric | Reference target | Derivation |
| --- | --- | --- |
| Sustained throughput | ~0.5 GB/s | 1 B/cycle × 500 MHz (LZ-only, level low). |
| Peak throughput (stretch) | ~2 GB/s | 4 parallel comparators (4 B/cycle). |
| Compression ratio (text) | 2.5–3.5× | PERF_BYTES_IN / PERF_BYTES_OUT. |
| Per-job latency (4 KB) | ~8–10 µs | fill(~64) + ~4096 compress + DMA cycles. |
| Cache hit-rate | > 95% | PERF_HITS / (HITS+MISS). |
| Huffman stage cost | ~10–25% throughput | Mode-dependent (LZ+Huffman vs LZ-only). |

Instrumentation: PERF_BYTES_IN/OUT, PERF_JOBS, PERF_CYCLES, PERF_HITS/MISS, and ARB_GRANTS are free-running (clear via CTRL / snapshot via DBG_CTRL.FREEZE_CTRS). Firmware computes GB/s = PERF_BYTES_IN / (PERF_CYCLES / f) and ratio = BYTES_IN / BYTES_OUT.

# 9  Error Handling

## 9.1  Detection and Response

Each block reports faults to the Control Module, which latches the first error code, captures context (ERR_ADDR, ERR_INFO), sets STATUS.ERROR, writes the error code into the job result, and raises the ERR interrupt (if enabled). CFG.HALT_ON_ERR chooses halt vs. skip-and-continue.

## 9.2  ERROR Register Bit-fields (0x040)

| Bits | Field | Condition |
| --- | --- | --- |
| [0] | ERR_DESC | Malformed job descriptor. |
| [1] | ERR_SRC_LEN | Zero or oversized source length. |
| [2] | ERR_DST_OVFL | Output exceeded dst_cap (after store fallback). |
| [3] | ERR_AXI | AXI SLVERR/DECERR on read/write. |
| [4] | ERR_ALIGN | Address/length alignment violation. |
| [5] | ERR_TIMEOUT | Watchdog expiry (no forward progress). |
| [6] | ERR_ECC | Uncorrectable ECC in history/FIFO SRAM. |
| [31:7] | Reserved | Reads 0. |

## 9.3  Recovery

- Firmware clears ERROR/INT_STATUS via W1C after reading ERR_ADDR/ERR_INFO.

- For fatal faults (ECC, timeout) firmware issues CTRL.SOFT_RESET before resuming.

- A watchdog guarantees a stuck job cannot hang the sub-system indefinitely.

# 10  Debug and Observability

## 10.1  DBG_CTRL Bit-fields (0x060)

| Bits | Field | Description |
| --- | --- | --- |
| [0] | HALT | Freeze the control FSM at the next job boundary. |
| [1] | STEP | Advance exactly one job while halted. |
| [2] | BREAK_ON_ERR | Auto-halt on any latched error. |
| [3] | FREEZE_CTRS | Atomically freeze performance counters for a consistent snapshot. |
| [31:4] | Reserved | Reads 0. |

## 10.2  Trace and Observation

- Job trace buffer: circular buffer of the last N jobs (job_id, in/out size, cycles, status), read one entry per DBG_TRACE access.

- DBG_STATE: live control-FSM state, accelerator sub-state, and active job_id.

- Observation bus: DBG_OBS_SEL routes a selected signal group (arbiter grants, cache hit/miss, FIFO levels, DMA state) to the optional debug port.

- DFT: scan-enable and per-SRAM MBIST provisions (history window, hash table, FIFOs) at integration level.

# 11  Clock and Reset Domains

- Single functional clock domain (datapath, control, CSR) in the reference design.

- Asynchronous-assert / synchronous-release reset; cache invalidate and FIFO clear before leaving reset.

- SOFT_RESET (CTRL[2]) resets datapath and FSM while preserving CSR configuration.

# 12  Parameters and Configurability

| Parameter | Default | Notes |
| --- | --- | --- |
| NUM_MASTERS | 3 | Input DMA, Output DMA, Accelerator. |
| WINDOW_BYTES | 32 KB | History-window size (CFG.WINDOW selectable). |
| MIN_MATCH | 3 | Minimum profitable match length. |
| MAX_MATCH | 258 | Maximum copy length (DEFLATE-class). |
| HASH_BITS | 15 | Hash-table index width. |
| CACHE_SIZE | 64 KB | sets × ways × line. |
| LINE_BYTES | 64 | Cache line / burst length. |
| RING_DEPTH | 256 | Job descriptor ring entries. |
| ADDR_WIDTH / DATA_WIDTH | 32 / 64 | AXI address / data width. |

# 13  Verification and Debug Hooks

- Round-trip self-check: hardware output decompressed by a reference model must match the input bit-exactly; CRC cross-checked.

- All PERF_*, ERROR, and DBG_* registers are runtime-readable for self-checking and the live dashboard.

- Directed error-injection per ERR_* class; randomized corpora for ratio/throughput stress.

- Assertions: AXI protocol per master, FIFO overflow/underflow, ring pointer monotonicity, history-window coherence, no-overflow-past-dst_cap.