Zephyr LZ Compression Offload Sub-System

High-level Architecture Specification (HAS)

Reference SoC Sub-System

| Document Type | High-level Architecture Specification (HAS) |
| --- | --- |
| Version | 1.0 (WIP) |
| Status | Work In Progress (WIP) |
| Date | September 4, 2026 |
| Author | Mayuresh Piske |

# Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-09-04 | M. Piske | Initial WIP draft for Hackathon 2026 submission. |

# Table of Contents

Revision History	2

Table of Contents	2

1  Introduction	4

1.1  Purpose	4

1.2  Scope	4

1.3  Intended Audience	4

1.4  References	4

1.5  Terminology and Abbreviations	4

2  Overview	5

2.1  Product Context and Application	5

2.2  How It Works (End-to-End)	5

2.3  Why This Architecture	5

2.4  System Context and Top-Level Block Diagram	6

3  Feature List	6

4  Architecture Overview	7

4.1  Block Responsibilities	7

5  External Interfaces	7

6  Operational Flow and Use Cases	8

6.1  Job Descriptor	8

6.2  Programming Sequence	8

6.3  Operating Modes	8

7  Functional Requirements	8

8  Clocking, Reset, and Power	9

9  Performance Numbers	9

10  Error Handling	9

11  Debug and Observability	10

12  Verification and Demo Strategy	10

13  Assumptions and Open Items	11

# 1  Introduction

## 1.1  Purpose

This High-level Architecture Specification (HAS) describes the architecture, feature set, interfaces, and application of the Zephyr LZ Compression Offload Sub-System (COSS). The COSS is a synthesizable, firmware-driven AXI sub-system that accepts a job (metadata plus a pointer to uncompressed data) from the host CPU, compresses the data in hardware using an LZ77-family algorithm, and writes the compressed result back to memory. It is the Hackathon 2026 reference design showcasing a complete SoC datapath — multi-master AXI interconnect with arbitration, a shared cache, a memory controller, a real application accelerator, and a firmware-visible control plane with error, performance, and debug facilities.

## 1.2  Scope

This document covers the architecture (HAS) view: what the sub-system does, its block partitioning, external interfaces, operational use cases, and functional/performance/error/debug requirements. Signal-level detail, the full register map, cache sizing, and internal FSMs are in the companion Micro-Architecture Specification (MAS).

## 1.3  Intended Audience

SoC architects, RTL and verification engineers, firmware/driver developers, and Hackathon reviewers.

## 1.4  References

- Zephyr COSS Micro-Architecture Specification (MAS), v0.1 – companion document.

- ARM AMBA AXI and ACE Protocol Specification (AXI4, AXI4-Lite).

- RFC 1951 – DEFLATE Compressed Data Format Specification (informative).

## 1.5  Terminology and Abbreviations

| Term | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface (AMBA AXI4) |
| AXI4-Lite | Lightweight, single-beat AXI variant used for register access |
| COSS | Compression Offload Sub-System (this IP) |
| LZ / LZ77 | Lempel-Ziv sliding-window compression algorithm family |
| DEFLATE | LZ77 + Huffman entropy coding (RFC 1951) |
| CSR | Control/Status Register (firmware-visible register space) |
| DMA | Direct Memory Access engine |
| FSM | Finite State Machine |
| QoS | Quality of Service (arbitration priority) |
| RR | Round-Robin (arbitration policy) |
| LRU | Least Recently Used (cache replacement policy) |
| WB | Write-Back (cache write policy) |
| W1C | Write-1-to-Clear register semantics |
| CRC | Cyclic Redundancy Check (integrity) |
| ECC | Error-Correcting Code (SRAM protection) |
| IRQ | Interrupt Request |
| HAS | High-level Architecture Specification |
| MAS | Micro-Architecture Specification |

# 2  Overview

## 2.1  Product Context and Application

The COSS is a scaled-down model of a hardware compression offload engine — the same class of IP found in storage controllers, SmartNICs, and datacenter accelerators (e.g. DEFLATE/LZ4 offload, Intel QAT-style engines). The host CPU offloads CPU-expensive compression to the COSS: it provides a small job descriptor (source address, length, destination address and capacity, algorithm/level) and the raw uncompressed data; the accelerator returns the compressed stream plus its size, a CRC, and a status.

End application: a firmware-programmable hardware compression accelerator. Firmware submits jobs; hardware performs LZ77 sliding-window compression (optionally followed by Huffman entropy coding, DEFLATE-style) and reports results and statistics. The same datapath supports a store (pass-through) fallback for incompressible data and is extensible to decompression as a future mode.

## 2.2  How It Works (End-to-End)

- Firmware places the uncompressed data in DRAM and builds a job descriptor (metadata: src/dst addresses, length, capacity, flags).

- Firmware writes the descriptor to the job ring and rings the doorbell (CTRL.START).

- The Control Module fetches the job; the Input DMA streams raw bytes into the accelerator.

- The LZ accelerator finds matches against the sliding-window history and emits literal / length-distance tokens (optionally Huffman-coded).

- The Output DMA writes the compressed stream to the destination buffer; the Control Module updates the descriptor with compressed size, CRC and status, and raises an interrupt.

- Firmware reads back the result and performance counters.

## 2.3  Why This Architecture

- Real accelerator with a strong cache story: the LZ history window / dictionary is re-read on every match search, so caching it (and the job descriptors) yields a very high hit-rate and a clear latency win — the cache genuinely earns its place.

- Multi-master AXI + arbiter: input-read DMA, output-write DMA, and accelerator history spill/fill all contend for DRAM; QoS keeps the output path moving so compression never stalls on write-back.

- Clean HW/FW contract: job descriptors, doorbell, status, interrupts, and error/perf/debug registers model exactly how a real driver interacts with an offload engine.

## 2.4  System Context and Top-Level Block Diagram

Figure 1 – Zephyr COSS top-level architecture and system context.

# 3  Feature List

| Feature | Description |
| --- | --- |
| LZ77 compression engine | Sliding-window match finder + literal/length-distance token encoder; optional Huffman stage (DEFLATE-class). |
| Configurable window / level | Firmware-selectable history-window size and effort level (match search depth). |
| Store fallback | Incompressible data is passed through in store mode to guarantee bounded output. |
| Job-descriptor interface | Firmware submits jobs via a descriptor ring with a doorbell; hardware writes back results. |
| AXI4 multi-master datapath | Input DMA, Output DMA, and accelerator masters sharing one memory subordinate. |
| QoS arbiter | Round-robin default with firmware-selectable priority mode. |
| L2 cache | Caches the history window and job descriptors; write-back, firmware-flushable. |
| Memory controller | AXI-to-DRAM command/bank FSM with refill/write-back buffering. |
| Output CRC | CRC over the compressed stream for end-to-end integrity. |
| Error handling | Classified error detection, capture, interrupt, and recovery (see §10). |
| Performance counters | Bytes in/out, compressed ratio, cycles, cache hits — for the live demo (see §9). |
| Debug facilities | State/job visibility, job trace buffer, breakpoints, counter snapshot (see §11). |

# 4  Architecture Overview

The COSS separates a datapath plane (interconnect, cache, memory controller) from a control plane (CSR register file and control module), with three masters (Input DMA, Output DMA, LZ accelerator) feeding the datapath and firmware driving the control plane.

## 4.1  Block Responsibilities

| Block | Responsibility |
| --- | --- |
| Input DMA (M0) | Reads uncompressed source data from DRAM and streams it into the accelerator input FIFO. |
| Output DMA (M1) | Writes the compressed stream from the accelerator output FIFO to the destination buffer. |
| LZ Accelerator (M2) | Match finder + token encoder + optional Huffman; maintains the history window; computes output CRC and size. |
| AXI Interconnect + Arbiter | Address-decodes and routes master traffic to the cache/memory subordinate; arbitrates contention (RR/QoS). |
| L2 Cache | Caches history window and job descriptors to cut DRAM latency; write-back with valid/dirty tracking. |
| Memory Controller | Serves cache misses/write-backs to DRAM via a bank/row command FSM. |
| CSR / Register Space | Firmware registers: job ring, control, config, status, error, performance, debug. |
| Control Module | Job sequencer FSM; decodes control/config, drives enables and arbiter policy, aggregates errors and IRQs. |

# 5  External Interfaces

| Interface | Type | Description |
| --- | --- | --- |
| Data / memory path | AXI4 (full) | Master read/write channels from the three masters into the interconnect; bursts for data and refill/writeback. |
| Configuration (CSR) | AXI4-Lite SUB | Firmware access to job ring and control/status/error/perf/debug registers; single-beat 32-bit. |
| Interrupt | Level IRQ | Aggregated done / error / threshold interrupt to the host CPU. |
| Clock / Reset | Signals | Single functional clock domain plus asynchronous-assert, synchronous-release reset (§8). |
| Debug port (optional) | Observation bus | Muxed internal-signal observation and trace read-out (§11). |

# 6  Operational Flow and Use Cases

## 6.1  Job Descriptor

Each job is described by a compact descriptor in the ring:

| Field | Meaning |
| --- | --- |
| src_addr | DRAM address of the uncompressed input buffer. |
| src_len | Length in bytes of the input. |
| dst_addr | DRAM address of the compressed output buffer. |
| dst_cap | Capacity of the output buffer (overflow guard). |
| flags | Algorithm (LZ-only / LZ+Huffman / store), effort level, window size. |
| job_id | Firmware tag echoed back in the result. |
| -- result (HW-written) -- | comp_size, crc32, status (OK / STORE / ERROR + code). |

## 6.2  Programming Sequence

- Firmware writes RING_BASE / RING_TAIL to register the job ring and its producer index.

- Firmware fills a descriptor and advances RING_TAIL (doorbell).

- Control Module consumes jobs from RING_HEAD until the ring is empty.

- On completion, hardware writes the result fields and raises DONE (or ERR) interrupt; firmware reads comp_size / status and PERF_* counters.

## 6.3  Operating Modes

- LZ-only: literal/length-distance tokens without entropy coding — fastest, lower ratio.

- LZ + Huffman (DEFLATE-class): adds the entropy stage for a higher ratio at some throughput cost.

- Store: automatic fallback when compression would exceed dst_cap, guaranteeing bounded output.

- Decompression (future): the same datapath can host an inflate engine as an added mode.

# 7  Functional Requirements

| ID | Requirement |
| --- | --- |
| FR-1 | The COSS shall compress a source buffer described by a job descriptor and write the result to the destination buffer. |
| FR-2 | The compressed output plus CRC shall be losslessly decompressible to the original input. |
| FR-3 | The engine shall fall back to store mode when compressed size would exceed dst_cap. |
| FR-4 | Firmware shall submit multiple jobs via a descriptor ring and receive per-job completion status. |
| FR-5 | The arbiter shall support round-robin and priority modes selectable at runtime. |
| FR-6 | The cache shall be transparent to correctness and firmware-flushable. |
| FR-7 | The sub-system shall detect, classify, capture, and report errors per §10. |
| FR-8 | Firmware shall be able to read performance and debug state at any time (§9, §11). |

# 8  Clocking, Reset, and Power

- Clocking: single functional clock (datapath + control + CSR) in the reference design; a CDC option is noted for multi-clock integration.

- Reset: asynchronous-assert / synchronous-release; the Control Module invalidates the cache and clears FIFOs before accepting jobs.

- Power: idle masters and the entropy stage are clock-gated when unused; no separate power domains in the reference scope.

# 9  Performance Numbers

Figures below are demonstrator estimates at a 500 MHz reference clock and a 32 KB history window; they are engineering targets, not production silicon guarantees.

| Metric | Reference target | Notes |
| --- | --- | --- |
| Sustained throughput | ~0.5 GB/s | 1 byte/cycle baseline match rate @ 500 MHz. |
| Peak throughput (stretch) | ~2 GB/s | 4 parallel match comparators (4 B/cycle). |
| Compression ratio (text) | 2.5–3.5× | DEFLATE-class on typical text/log data. |
| Compression ratio (binary) | 1.3–2.0× | Mixed binary; store fallback at 1.0×. |
| Per-job latency (4 KB) | ~8–10 µs | Pipeline fill + compress + DMA at 500 MHz. |
| History/descriptor hit-rate | > 95% | Window fits in cache after warm-up. |
| Pipeline fill latency | ~64 cycles | Input FIFO → first token. |

Throughput model: throughput ≈ f × (bytes advanced per cycle). The baseline advances one byte per cycle during literal/short-match processing; deeper match search or the Huffman stage can lower this, while parallel comparators raise it. The live demo plots bytes-in and bytes-out over time and derives GB/s and ratio from PERF counters.

# 10  Error Handling

Errors are detected per job, classified, captured with context, reported through the ERROR register and an interrupt, and recovered by firmware. On error the current job is aborted; behavior for subsequent jobs is controlled by CFG.HALT_ON_ERR (halt vs. skip-and-continue).

| Error | Cause / handling |
| --- | --- |
| ERR_DESC | Malformed job descriptor (bad flags, null pointers) → abort job, ERR IRQ. |
| ERR_SRC_LEN | Zero or oversized src_len → abort job. |
| ERR_DST_OVFL | Compressed size would exceed dst_cap → auto store-fallback, or abort if store also overflows. |
| ERR_AXI | AXI SLVERR/DECERR on a read or write beat → abort, capture address. |
| ERR_ALIGN | Address/length alignment violation → abort job. |
| ERR_TIMEOUT | Watchdog expiry (job made no progress) → abort, flag hang. |
| ERR_ECC | Uncorrectable ECC fault in history/FIFO SRAM → abort, mark data corruption. |

- Capture: the ERROR register latches the first error code; a companion ERR_ADDR/ERR_INFO captures the faulting address and job_id.

- Reporting: STATUS.ERROR is set, the job's result status carries the code, and an ERR interrupt is raised if enabled.

- Recovery: firmware clears the error (W1C), optionally issues SOFT_RESET, and re-submits or skips the job.

# 11  Debug and Observability

- State visibility: DBG_STATE exposes the control FSM state, active job_id, and accelerator sub-state for polling.

- Job trace buffer: a circular buffer logs the last N completed jobs (job_id, in/out size, cycles, status) for post-run analysis.

- Breakpoints / single-step: DBG_CTRL supports halt, single-job step, and break-on-error to freeze the pipeline for inspection.

- Counter snapshot: performance counters can be frozen atomically so firmware reads a consistent set.

- Observation bus: DBG_OBS_SEL muxes internal signal groups (arbiter grants, cache hit/miss, FIFO levels) to an optional debug port.

- DFT hooks: scan-enable and SRAM MBIST provisions for manufacturing test (integration-level).

# 12  Verification and Demo Strategy

- Round-trip check: compress in hardware, decompress in a reference model, and assert bit-exact recovery of the input.

- Corpus tests across text, log, and binary data to exercise ratio and store fallback; directed error-injection tests for each ERR_* class.

- Live dashboard reads PERF_* and DBG_* over the CSR interface to plot throughput, ratio, cache hit-rate, and per-job latency during the demo.

- Optional FPGA target compresses a real file end-to-end for a Gold-level live demo.

# 13  Assumptions and Open Items

- LZ77 sliding-window core with an optional static-Huffman stage in the reference scope; dynamic Huffman and larger windows are extensions.

- Single-stream (single active job) compression; multi-context/multi-queue is future work.

- Simplified fixed-latency DRAM model; full DDR PHY/timing closure is out of scope.

- No security hardening or power/timing sign-off in the Hackathon deliverable.